# WELL REGISTRATION ATTACHMENT

# (To be handed out to all Applicants)

| Criteria | EXEMPT WELL | NON-EXEMPT WELL |
| --- | --- | --- |
| Permitted Use | Restricted: Must be for Domestic, Livestock, Poultry, or Wildlife use only. | Unrestricted: Any beneficial use (e.g., irrigation, commercial, industrial) is allowed if permitted. |
| Pump Capacity | ≤ 25,000 gallons/day (17.36 GPM) | > 25,000 gallons/day (>17.36 GPM) |
| Operating Permit Required? | No | Yes |
| Application Fee | $500 | $3,000 (new wells) |
| Annual Production Reports | Not required | Required by Jan 31st each year |

### EXEMPT WELL USE DEFINITIONS (Rule 2 & 3.1)

To qualify as Exempt, the well must serve one of these specific purposes:

- Domestic: Personal/household needs (drinking, bathing, cooking), irrigation of lawns/family gardens, swimming pools, and watering domestic animals.

- Livestock: Water for farm/ranch animals (cattle, horses, sheep, poultry, etc.).

- Poultry/Wildlife: Watering for poultry operations or wildlife conservation.

- Pond Limit: Exempt wells may fill ponds/tanks up to 50,000 gallons only for the above uses.

### KEY REQUIREMENTS FOR ALL WELLS

BEFORE DRILLING:

☐ Register with District (new wells require Well Drilling Authorization)

☐ Notify District prior to drilling (Rule 4.1.A)

☐ Ensure well spacing compliance (see table below)

AFTER COMPLETION:

☐ Submit State of Texas Well Report within 60 days

☐ Submit Pump Installation Report within 60 days

☐ Report any change in well condition, operation, or status

### WELL SPACING REQUIREMENTS (New Wells After Feb 11, 2002)

| Pumping Capability | Distance Between Wells | Distance from Property Lines |
| --- | --- | --- |
| 17.36 GPM or less | 100 feet | 50 feet |
| 17.37 – 50.00 GPM | 300 feet | 150 feet |
| 50.01 – 100.00 GPM | 750 feet | 375 feet |
| 100.01 – 300.00 GPM | 1,200 feet | 600 feet |
| Over 300.00 GPM | 1,500 feet | 750 feet |

SEPTIC SYSTEM SPACING (State Requirements):

- 50 feet minimum from Septic Tanks (16 TAC § 76.1000).

- 100 feet minimum from Septic Disposal Fields (16 TAC § 76.1000).

### SPECIAL POND STORAGE RULES

| Well Type | Storage Allowance | Restrictions |
| --- | --- | --- |
| Exempt Wells | Up to 50,000 gallons | Domestic use or livestock watering only. Additional water must come from rainfall/runoff. |
| Non-Exempt Wells | Generally prohibited | Exception: Operating Permit may authorize temporary storage for non-domestic irrigation systems with boost pumps. |

Note: During Drought Stages 3 & 4, no groundwater may be pumped into ponds, lakes, or reservoirs for any purpose, even for livestock. Water must come from rainfall or runoff only.

### IMPORTANT NOTICES

⚠️ Change in Status: If you modify an exempt well to increase withdrawal rate, you may be reclassified as non-exempt and required to obtain an Operating Permit.

⚠️ Change of Ownership: Must be reported to District within 90 days of ownership transfer.

⚠️ Export Permits: Required for transferring groundwater outside District boundaries (Rule 3.6).

⚠️ Aquifer Restrictions: Operating Permits will NOT be issued for new wells withdrawing from Edwards-Trinity (Plateau), Upper Glen Rose (Upper Trinity), or Marble Falls Aquifers (Rule 3.1.C).

⚠️ Test Wells: Exempt from Operating Permit but must be properly plugged or converted with permit after testing concludes.

Updated: May 2026 | Based on BPGCD Rules Adopted April 16, 2026 (Board Resolution 20260416-5)